AT&T IP Flexible Reach Service with Enhanced Features Using MIS / PNT or AT&T Virtual Private Network Transport with Cisco Unified Communications Manager v. 11.0 and Cisco UBE v. 11.1.0 on an ISR G2 Router with IPv6 SIP Interface

MAR 2016

Table of Contents

Introduction	4

Network Topology	5

Hardware Components	6

Software Requirements	6

Features	7

Features – Supported	7

Network Based Features - Supported	7

Features - Not Supported	7

Caveats	8

Auto-Attendant	8

Hold/Resume & Music on Hold (MOH)	8

PBX Based Call Forward Unconditional	8

SIP Provisional Acknowledgement/Early media	8

AT&T IP Teleconferencing (IPTC)	8

Configuration Considerations	9

Emergency 911/E911 Services Limitations and Restrictions	9

ISR Configuration	10

Cisco UCM Configuration	32

Cisco UCM Version	32

Cisco UCM Audio Codec Preference List	33

Cisco UCM Region Configuration	34

Device Pool Configuration	35

Annunciator Configuration	38

Conference Bridge Configuration	39

Media Termination Point Configuration	40

Music on Hold Server Configuration	41

Music on Hold Service (IP Voice Media Streaming App) Parameter Settings	42

Music on Hold Service (Duplex Streaming) Parameter Settings	43

Media Resource Group Configuration	44

Media Resource Group List Configuration	45

UC Service Configuration	46

Service Profile Configuration	49

End User Configuration	51

Cisco IP Phone 7975 SIP Configuration	55

Cisco IP Phone 9971 SIP Configuration	66

SIP Trunk Security Profile Configuration used by SIP trunk to Cisco UBE	78

SIP Profile Configuration used by SIP trunk to Cisco UBE	79

SIP Trunk to Cisco UBE Configuration	83

Route Pattern Configuration	91

Jabber Client Configuration	98

Voicemail Port Configuration	104

Voicemail Pilot Configuration	106

FAX Gateway Configuration	107

Cisco UCM SCCP Integration with Cisco Unity Connection (CUC)	124

CUC Version	124

CUC Telephony Integration with Cisco UCM	125

CUC Port Group	126

CUC Port Settings	128

CUC Sample User Basic Settings	129

Auto Attendant	131

Cisco UCM Integration with Cisco Unified CM IM and Presence (CUP/IMP)	133

CUP/IMP Version	134

Presence Topology	134

Node Configuration	135

Users	136

Presence gateway configuration	137

Acronyms	139

Important Information	140

# Introduction

Service Providers today, such as AT&T, are offering alternative methods to connect to the PSTN via their IP network. Most of these services utilize SIP as the primary signaling method and a centralized IP to TDM gateway to provide on-net and off-net services. AT&T IP Flexible Reach is a service provider offering that allows connection to the PSTN and may offer the end customer a viable alternative to traditional PSTN connectivity via either analog or T1 lines. A demarcation device between these services and customer owned services is recommended. The Cisco Unified Border Element (Cisco UBE) provides demarcation, security, interworking and session management services.

- This application note describes the necessary steps and configurations of Cisco Unified Communications Manager (Cisco UCM) 11.0, Cisco Unity Connection 11.0, Cisco Unified CM IM and Presence 11.0, Cisco Integrated Services Routers (ISR) Version 15.5(3)M1 with connectivity to AT&T’s IP Flex-Reach IPV6 SIP trunk service. It also covers support and configuration example of Cisco Unity Connection (CUC) messaging integrated with Cisco Unified Communications Manager (Cisco UCM). The deployment model covered in this application note is Cisco Integrated Services Routers (ISR) to PSTN (AT&T IP Flexible Reach SIP). AT&T IP Flexible Reach provides inbound and outbound call service.

- Testing was performed in accordance to AT&T’s IP Flexible Reach test plan and all features were verified. Key features verified are: inbound and outbound basic call (including international calls), calling name delivery, calling number and name restriction, CODEC negotiation, intra-site transfers, intra-site conferencing, call hold and resume, call forward (forward all, busy and no answer), leaving and retrieving voicemail (Cisco Unity Connection), CISCO auto-attendant (BACD), fax G.711 and T38 (G3 and SG3 speeds), teleconferencing, failover of unresponsive SIP network to PSTN and outbound/inbound calls to/from TDM networks.

- The Cisco Unified Border Element function configuration detailed in this document is based on a lab environment with a simple dial-plan used to ensure proper interoperability between AT&T SIP network and Cisco Integrated services router. The configurations described in this document details the important commands for successful interoperability. Care must be taken by the network administrator deploying Cisco ISR to ensure these commands are set per each dial-peer required, to interoperate done AT&T SIP network.

- Consult your Cisco representative for the correct IOS image and for the specific application and Device Unit License and Feature License requirements for all your Cisco Unified Communication Manager with Cisco Unified Border Element components.

# Network Topology

# Hardware Components

- UCS-C240 VMWare server running ESXi 5.5

- Cisco IP Phones. This solution was tested with Cisco 7975 & Cisco 9971 phones

- Cisco CISCO2921/K9 (revision 1.0) with 483328K/40960K bytes of memory.

- Processor board ID FTX1746AJCB

- 3 Gigabit Ethernet interfaces

255K bytes of non-volatile configuration memory.

# Software Requirements

- Cisco UCM: System version: 11.0.1.10000-10, including Business Edition 6000 and Business Edition 7000.

- ISR: Cisco IOS Software, C2900 Software (C2900-UNIVERSALK9_NPE-M), Version 15.5(3)M1, RELEASE SOFTWARE (fc2)

- Cisco UBE Software Release 11.1.0

- System image file is “c2900-universalk9_npe-mz.SPA.155-3.M1.bin".

- Cisco Unity Connection version: System version: 11.0.1.10000-10

- Cisco Unified CM IM and Presence: System version: 11.0.1.10000-6

- Cisco Jabber client version: 11.0.0 Build 65527

- VentaFax client version: 7.6.244.598 I

# Features

## Features – Supported

- Basic Call using G.729 and G711

- Calling Party Number Presentation and Restriction

- Calling Name Presentation

- AT&T Advanced 8YY Call Prompter (8YY)

- Cisco UBE Delayed-Offer-to-Early-Offer conversion of an initial SIP INVITE without SDP

- Intra-site Call Transfer

- Intra-site Conference

- Call Hold and Resume

- Call Forward All, Busy and No Answer

- AT&T IP Teleconferencing

- Fax over G.711

- Fax using T.38

- Incoming DNIS Translation and Routing

- Outbound calls to AT&T’s IP and TDM networks

- Inbound calls from AT&T’s IP and TDM networks

- CPE voicemail managed service, leave and retrieve voice messages via incoming AT&T SIP trunk (Cisco Unity Connection)

- Auto-attendant transfer-to service (See Caveat section for details)

- Failover (From non-responsive SIP network to ATT SIP network)

- Inbound & Outbound Calls using Cisco Jabber

- Emergency and 411 calls were terminated to a voicemail platform in lab environment within AT&T for test

- RTCP

## Network Based Features - Supported

- Call forward (Unconditional, Busy, No Answer, Not reachable)

- Sequential Ringing

- Simultaneous Ringing

NOTE:  Using the AT&T IP Flexible Reach Portal, provision TN(s) on the CPE with the Sequential Ring and simultaneous feature. Provisioning is self-explanatory.  Please contact your AT&T representative, if you need help with the provisioning Network based feature.

## Features - Not Supported

- Cisco UCM Codec negotiation of G.722.1

- Network-Based Blind Call Transfer

- Network-Based Consultative Call Transfer

# Caveats

## Auto-Attendant

- The CUC auto-attendant feature was used to test attendant functionality using the default codec G711 for auto attendant prompts. G729 prompts can be used but was not tested.

## Hold/Resume & Music on Hold (MOH)

- Re-invites for hold/resume from PSTN network is potentially dependent on the carrier/network through which the call is traversing.

## PBX Based Call Forward Unconditional

- PBX Based Unconditional Call Forwarding test is temporarily blocked due to AT&T Flexible Reach network issue.

## Operator Call

- Operator Call (Call to 0) is not working in AT&T VIT lab.

## SIP Provisional Acknowledgement/Early media

- To play early media sent by ATT, Cisco UCM needs to be enabled with PRACK if 1XX contains SDP on Cisco UCM SIP Profile.

- Some PSTN network call prompters utilize early-media cut-through to offer menu options to the caller (DTMF select menu) before the call is connected. In order for Cisco UCM/Cisco UBE solution to achieve successful early-media cut-through, the Cisco UCM to Cisco UBE call leg must be enabled with SIP PRACK. To enable SIP PRACK on the Cisco UCM, the SIP Profile “SIP Rel1XX Options” setting must be set to “Send PRACK”. The SIP Profile is found under Device>Device Settings>SIP Profile, This feature can be assigned on a per SIP trunk basis using SIP profiles. SIP PRACK provisioning on Cisco UCM 9.X and newer software versions is enabled under SIP Profile configuration page, while SIP PRACK support on Cisco UCM 7.X and older software versions is enabled under the Service Parameters configuration page.

## AT&T IP Teleconferencing (IPTC)

Following scenarios were not executed due to limitations on AT&T network

- IPTC - Hold & Resume

- IPTC - PBX-Based Attended Transfer

- IPTC - PBX-Based 3-way Call Conference

# Configuration Considerations

- To enable conference on AT&T IP Flexible Reach and Cisco UCM SIP trunk, it is required to configure a conference bridge (CFB) resource to initiate a three-way conference between end-points. See configuration section for details.

- Forwarded calls from Cisco UCM user to PSTN (out to AT&T’s IP Flexible Reach service), AT&T serviced areas require that the SIP Diversion header contain the full 10-digit DID number of the forwarding party. In this application note the assumption has been made that a typical customer will utilize extension numbers (4-digit assignments in this example) and map 10-digit DID number using Cisco UBE translation profile.  This is because the Cisco UCM uses 4-digit extensions on Cisco UCM IP phones and it is necessary to expand the 4-digit extension included in the Diversion header of a forwarding INVITE message to its full 10-digit DID number when the IP phone is set to call-forward. The requirement to expand the Diversion-Header has been achieved by the use of a SIP profile in Cisco UBE (See configuration section for details).

- Upon receiving inbound calls, AT&T SIP network will always have the first choice codec presented in the initial SIP INVITE (unless the end-device does not support the listed preferred codec), and processes calls accordingly. Customers wishing to place/receive G.711-only calls must configure separate voice class codec on Cisco UBE with G.711 as the first choice.

- SIP Profiles may also be employed to advertise desired RTP payload packet size.

- “voice-class sip privacy id” needs to configure in Cisco UBE dial peer to make call From a CPE Phone to PSTN phone, Pass Calling Party Number (CPN), marked private.

- This test environment is not configured with Cisco UBE High Availability (HA)

- Cisco UCM sends a SIP UPDATE message to Cisco UBE for a call transfer.  AT&T network does not support the SIP UPDATE message causing the Cisco UBE to timeout and the call transfer is not completed.  As a workaround, SIP profile has been applied on the Cisco UBE to remove UPDATE from the allowed headers (See configuration section for details).

# Emergency 911/E911 Services Limitations and Restrictions

- Emergency 911/E911 Services Limitations and Restrictions - Although AT&T provides 911/E911 calling capabilities, AT&T does not warrant or represent that the equipment and software (e.g., IP PBX) reviewed in this customer configuration guide will properly operate with AT&T IP Flexible Reach to complete 911/E911 calls; therefore, it is Customer's responsibility to ensure proper operation with its equipment/software vendor

- While AT&T IP Flexible Reach services support E911/911 calling capabilities under certain Calling Plans, there are circumstances when E911/911 service may not be available, as stated in the Service Guide for AT&T IP Flexible Reach found at http://new.serviceguide.att.com. Such circumstances include, but are not limited to, relocation of the end user’s CPE, use of a non-native or virtual telephone number, failure in the broadband connection, loss of electrical power and delays that may occur in updating the Customer’s location in the automatic location information database. Please review the AT&T IP Flexible Reach Service Guide in detail to understand the limitations and restrictions

# ISR Configuration

ATT-IPV6#show version

Cisco IOS Software, C2900 Software (C2900-UNIVERSALK9_NPE-M), Version 15.5(3)M1, RELEASE SOFTWARE (fc2)

Technical Support: http://www.cisco.com/techsupport

Copyright (c) 1986-2015 by Cisco Systems, Inc.

Compiled Mon 16-Nov-15 19:25 by prod_rel_team

ROM: System Bootstrap, Version 15.0(1r)M16, RELEASE SOFTWARE (fc1)

ATT-IPV6 uptime is 2 weeks, 5 days, 23 hours, 6 minutes

System returned to ROM by reload at 07:05:03 UTC Wed Mar 9 2016

System image file is "flash:c2900-universalk9_npe-mz.SPA.155-3.M1.bin"

Last reload type: Normal Reload

Last reload reason: Reload Command

This product contains cryptographic features and is subject to United

States and local country laws governing import, export, transfer and

use. Delivery of Cisco cryptographic products does not imply

third-party authority to import, export, distribute or use encryption.

Importers, exporters, distributors and users are responsible for

compliance with U.S. and local country laws. By using this product you

agree to comply with applicable laws and regulations. If you are unable

to comply with U.S. and local laws, return this product immediately.

A summary of U.S. laws governing Cisco cryptographic products may be found at:

http://www.cisco.com/wwl/export/crypto/tool/stqrg.html

If you require further assistance please contact us by sending email to

export@cisco.com.

Cisco CISCO2921/K9 (revision 1.0) with 483328K/40960K bytes of memory.

Processor board ID FTX1746AJCB

3 Gigabit Ethernet interfaces

1 terminal line

DRAM configuration is 64 bits wide with parity enabled.

255K bytes of non-volatile configuration memory.

250880K bytes of ATA System CompactFlash 0 (Read/Write)

License Info:

License UDI:

-------------------------------------------------

Device#   PID                   SN

-------------------------------------------------

*1        CISCO2921/K9          FTX1746AJCB

Suite License Information for Module:'c2900'

--------------------------------------------------------------------------------

Suite                 Suite Current         Type           Suite Next reboot

--------------------------------------------------------------------------------

FoundationSuiteK9_npe None                  None           None

securityk9_npe

datak9

AdvUCSuiteK9          None                  None           None

uck9

cme-srst

cube

Technology Package License Information for Module:'c2900'

------------------------------------------------------------------------

Technology    Technology-package                  Technology-package

Current              Type           Next reboot

------------------------------------------------------------------------

ipbase        ipbasek9             Permanent      ipbasek9

security      None                 None           None

uc            uck9                 Permanent      uck9

data          None                 None           None

Configuration register is 0x2102

ATT-IPV6#sh running-config

Building configuration...

Current configuration : 11224 bytes

!

! Last configuration change at 06:08:57 UTC Thu Mar 17 2016 by cisco

!

version 15.5

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

service sequence-numbers

!

hostname ATT-IPV6

!

boot-start-marker

boot system flash c2900-universalk9_npe-mz.SPA.155-3.M1.bin

boot-end-marker

!

aqm-register-fnf

!

logging queue-limit 1000000000

logging buffered 30000000

logging rate-limit 10000

no logging console

no logging monitor

enable secret 4 Pe0NhiWw5IXZpE.k5VhTSCoGPcuVeRyrer9kEPz20Z6

!

no aaa new-model

ethernet lmi ce

!

!

!

!

!

!

!

!

!

!

!

!

no ip domain lookup

ip cef

ipv6 unicast-routing

ipv6 cef

multilink bundle-name authenticated

!

!

!

!

!

cts logging verbose

!

!

voice-card 0

dspfarm

dsp services dspfarm

!

!

!

voice service voip

no ip address trusted authenticate

address-hiding

mode border-element

allow-connections sip to sip

redirect ip2ip

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

no fax-relay sg3-to-g3

sip

header-passing

error-passthru

no update-callerid

early-offer forced

midcall-signaling passthru

privacy-policy passthru

g729 annexb-all

!

voice class codec 2

codec preference 1 g711ulaw

codec preference 2 g729r8 bytes 30

!

voice class codec 3

codec preference 1 g711ulaw

!

voice class codec 1

codec preference 1 g729r8 bytes 30

codec preference 2 g711ulaw

!

!

voice class sip-profiles 1

response ANY sip-header Allow-Header modify "UPDATE," ""

request INVITE sip-header Diversion modify "<sip:(.*)@(.*)>" "<sip:732216\1@\2>"

request INVITE sdp-header Audio-Attribute modify "a=ptime:20" "a=ptime:30"

response ANY sdp-header Audio-Attribute modify "a=ptime:20" "a=ptime:30"

request INVITE sdp-header Audio-Attribute add "a=ptime:30"

!

!

!

!

!

voice translation-rule 1

rule 1 /^.*\(435.\)/ /732320\1/

!

!

!

voice translation-profile NPA

translate calling 1

!

!

!

license udi pid CISCO2921/K9 sn FTX1746AJCB

hw-module pvdm 0/0

!

!

!

username cisco privilege 15 password 0 cisco

!

redundancy

!

!

!

!

!

!

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

interface GigabitEthernet0/0

description WAN to ATT

no ip address

duplex auto

speed auto

ipv6 address 2620:96:C000:4::10/64

ipv6 enable

!

interface GigabitEthernet0/1

description LAN Interface

ip address 10.80.14.5 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/2

no ip address

shutdown

duplex auto

speed auto

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip route 0.0.0.0 0.0.0.0 192.65.79.33

ip route 10.64.0.0 255.255.0.0 10.80.14.1

ip route 10.64.0.0 255.255.0.0 10.80.22.1

ip route 10.80.0.0 255.255.0.0 10.80.14.1

ip route 172.16.0.0 255.255.0.0 10.80.14.1

!

ipv6 route ::/0 2620:96:C000:4::1

!

!

!

control-plane

!

!

!

!

!

!

mgcp behavior rsip-range tgcp-only

mgcp behavior comedia-role none

mgcp behavior comedia-check-media-src disable

mgcp behavior comedia-sdp-force disable

!

mgcp profile default

!

!

!

!

dial-peer voice 200 voip

description "Outgoing To AT&T .IP PBX facing side"

session protocol sipv2

incoming called-number [2-9][0-9]........

voice-class codec 1

voice-class sip asymmetric payload full

voice-class sip asserted-id pai

voice-class sip privacy-policy passthru

voice-class sip bind control source-interface GigabitEthernet0/1

voice-class sip bind media source-interface GigabitEthernet0/1

dtmf-relay rtp-nte

no fax-relay sg3-to-g3

fax rate 14400

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

no vad

!

dial-peer voice 214 voip

description "Outgoing To AT&T"-AT&T facing side

destination-pattern [2-9][0-9]........

session protocol sipv2

session target ipv6:[2001:1890:1253:0:12:38:172:80]

voice-class codec 1

voice-class sip asymmetric payload full

voice-class sip asserted-id pai

voice-class sip privacy-policy passthru

voice-class sip early-offer forced

voice-class sip profiles 1

voice-class sip bind control source-interface GigabitEthernet0/0

voice-class sip bind media source-interface GigabitEthernet0/0

dtmf-relay rtp-nte

no fax-relay sg3-to-g3

fax rate 14400

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

no vad

!

dial-peer voice 800 voip

description " Incoming AT&T to IP-PBX . AT&T facing side "

translation-profile incoming test+1

huntstop

session protocol sipv2

incoming called-number [37][13][24][12]......

voice-class codec 1

voice-class sip asymmetric payload full

no voice-class sip asserted-id

voice-class sip privacy-policy passthru

voice-class sip profiles 1

voice-class sip bind control source-interface GigabitEthernet0/0

voice-class sip bind media source-interface GigabitEthernet0/0

dtmf-relay rtp-nte

no fax-relay sg3-to-g3

fax rate 14400

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

no vad

!

dial-peer voice 700 voip

description " Incoming AT&T to IP-PBX - IP-PBX facing side "

huntstop

destination-pattern [37][13][24][12]......

no modem passthrough

session protocol sipv2

session target ipv4:10.80.14.2:5060

voice-class codec 1

voice-class sip asymmetric payload full

no voice-class sip asserted-id

voice-class sip privacy-policy passthru

voice-class sip early-offer forced

voice-class sip profiles 1

voice-class sip bind control source-interface GigabitEthernet0/1

voice-class sip bind media source-interface GigabitEthernet0/1

dtmf-relay rtp-nte

no fax-relay sg3-to-g3

fax rate 14400

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

no vad

!

dial-peer voice 100 voip

description "Outgoing To AT&T"-AT&T facing side

destination-pattern 73236.....

no modem passthrough

session protocol sipv2

session target ipv6:[2001:1890:1253:0:12:38:172:80]

voice-class codec 1

voice-class sip asymmetric payload full

voice-class sip asserted-id pai

voice-class sip privacy-policy passthru

voice-class sip profiles 1

voice-class sip bind control source-interface GigabitEthernet0/0

voice-class sip bind media source-interface GigabitEthernet0/0

dtmf-relay rtp-nte

fax rate 14400

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

no vad

!

dial-peer voice 300 voip

description " Int'l calls to AT&T - AT&T facing side "

destination-pattern 011T

no modem passthrough

session protocol sipv2

session target ipv6:[2001:1890:1253:0:12:38:172:80]

voice-class codec 1

voice-class sip asymmetric payload full

voice-class sip asserted-id pai

voice-class sip privacy-policy passthru

voice-class sip early-offer forced

voice-class sip profiles 1

voice-class sip bind control source-interface GigabitEthernet0/0

voice-class sip bind media source-interface GigabitEthernet0/0

dtmf-relay rtp-nte

fax rate 14400

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

no vad

!

dial-peer voice 400 voip

description " Int'l calls to AT&T - IP-PBX facing side "

no modem passthrough

session protocol sipv2

incoming called-number 011T

voice-class codec 1

voice-class sip asymmetric payload full

voice-class sip asserted-id pai

voice-class sip privacy-policy passthru

voice-class sip profiles 1

voice-class sip bind control source-interface GigabitEthernet0/1

voice-class sip bind media source-interface GigabitEthernet0/1

dtmf-relay rtp-nte

fax rate 14400

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

no vad

!

dial-peer voice 500 voip

description " N11 Calls to AT&T - AT&T facing side "

destination-pattern .11

no modem passthrough

session protocol sipv2

session target ipv6:[2001:1890:1253:0:12:38:172:80]

voice-class codec 1

voice-class sip asymmetric payload full

voice-class sip asserted-id pai

voice-class sip privacy-policy passthru

voice-class sip early-offer forced

voice-class sip profiles 1

voice-class sip bind control source-interface GigabitEthernet0/0

voice-class sip bind media source-interface GigabitEthernet0/0

dtmf-relay rtp-nte

fax rate 14400

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

no vad

!

dial-peer voice 600 voip

description " N11 Calls to AT&T - IP-PBX facing side "

no modem passthrough

session protocol sipv2

incoming called-number .11

voice-class codec 1

voice-class sip asymmetric payload full

voice-class sip asserted-id pai

voice-class sip privacy-policy passthru

voice-class sip profiles 1

voice-class sip bind control source-interface GigabitEthernet0/1

voice-class sip bind media source-interface GigabitEthernet0/1

dtmf-relay rtp-nte

fax rate 14400

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

no vad

!

dial-peer voice 122 voip

description "OPERATOR TESTING"

destination-pattern 0

no modem passthrough

session protocol sipv2

session target ipv6:[2001:1890:1253:0:12:38:172:80]

voice-class codec 1

voice-class sip asymmetric payload full

voice-class sip asserted-id pai

voice-class sip privacy-policy passthru

voice-class sip early-offer forced

voice-class sip profiles 1

voice-class sip bind control source-interface GigabitEthernet0/0

voice-class sip bind media source-interface GigabitEthernet0/0

dtmf-relay rtp-nte

fax rate 14400

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

no vad

!

dial-peer voice 141 voip

description "Network Feature"

translation-profile outgoing NPA

destination-pattern *..

no modem passthrough

session protocol sipv2

session target ipv6:[2001:1890:1253:0:12:38:172:80]

voice-class codec 1

voice-class sip asymmetric payload full

voice-class sip asserted-id pai

voice-class sip privacy-policy passthru

voice-class sip early-offer forced

voice-class sip profiles 1

voice-class sip bind control source-interface GigabitEthernet0/0

voice-class sip bind media source-interface GigabitEthernet0/0

dtmf-relay rtp-nte

fax rate 14400

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

no vad

!

dial-peer voice 2151 voip

description "Outgoing To AT&T"-AT&T facing side

destination-pattern 7323204607

no modem passthrough

session protocol sipv2

session target ipv6:[2001:1890:1253:0:12:38:172:80]

voice-class codec 1

voice-class sip asymmetric payload full

voice-class sip asserted-id pai

voice-class sip privacy-policy passthru

voice-class sip early-offer forced

voice-class sip profiles 1

voice-class sip bind control source-interface GigabitEthernet0/0

voice-class sip bind media source-interface GigabitEthernet0/0

dtmf-relay rtp-nte

fax rate 14400

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

no vad

!

!

gateway

media-inactivity-criteria all

timer receive-rtcp 5

timer receive-rtp 86400

!

sip-ua

no remote-party-id

retry invite 2

timers expires 1800000

connection-reuse

protocol mode dual-stack

!

!

!

gatekeeper

shutdown

!

!

!

line con 0

logging synchronous

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport output pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1

line vty 0 4

exec-timeout 960 0

logging synchronous

login local

transport input all

!

scheduler allocate 20000 1000

!

end

# Cisco UCM Configuration

The configuration screen shots shows general over view of lab configuration for this interoperability testing.

## Cisco UCM Version

## Cisco UCM Audio Codec Preference List

Navigation Path: System  Region Information  Audio codec preference list

Cisco UCM 11.0 has a feature called Audio Codec Preference List. This feature allows to configure the order of audio codec preference both for Inter and Intra Region calls. Audio Codec Preference list is assigned to the Region used by the Device Pool for Phones and by Conference Bridges. Based on user requirement, different codec regions can be assigned as their first choice codec with this configuration for inbound calls as well as conferences initiated by Cisco IP phones. Audio codec preference for outbound calls is determined by Cisco UBE (via configuration of voice-class codec)

## Cisco UCM Region Configuration

Navigation Path: System  Region Information  Region

## Device Pool Configuration

Navigation Path: System  Device Pool

“G729” Device Pool is configured for testing the interoperability. No special consideration needs to be taken when configuring the Device Pools. Optionally, a Media Resource Group List can be added to the Device Pools, if needed, to assign selected Media Resources (Conference Bridges, Transcoders, MoH servers, Annunciators) to devices.

Device Pool Configuration (continued…)

Device Pool Configuration (continued…)

## Annunciator Configuration

Navigation: Media Resource  Annunciator

Set Name* = ANN_2.

Set Description = ANN_clus24pubsub. This is used for this example

Set Device Pool* = G729.

## Conference Bridge Configuration

Navigation: Media Resources  Conference Bridge

Set Conference Bridge Type* = Cisco Conference Bridge Software.

Set Host Server = clus24pubsub. This is used for this example.

Set Conference Bridge Name* = CFB_2.

Set Description = CFB_clus24pubsub. This is used in this example.

Set Device Pool* = G729.

## Media Termination Point Configuration

Navigation: Media ResourceMedia Termination Point

Set Media Termination Point Name* = MTP_2

Set Description = MTP_clus24pubsub. This is used for this example

Set Device pool* = G729

## Music on Hold Server Configuration

Navigation: Media Resources  Music on Hold Server

Set Music on Hold Server Name* = MOH_2.

Set Description = MOH_clus24pubsub. This is used for this example.

Set Device Pool* = G729

## Music on Hold Service (IP Voice Media Streaming App) Parameter Settings

Navigation: System  Service Parameter

Note: Make sure codecs G.729 Annex A and G.711 mulaw are configured in parameter Supported MOH Codecs.

Select Server* = clus24pubsub--CUCM Voice/Video (Active). This is used in this example.

Select Service* = Cisco IP Voice Media Streaming App (Active

## Music on Hold Service (Duplex Streaming) Parameter Settings

Navigation: System  Service Parameter

Select Server* = clus24pubsub--CUCM Voice/Video (Active). This is used in this example.

Select Service* = Cisco CallManager (Active).

Select Duplex Streaming Enabled * = True

## Media Resource Group Configuration

Navigation Path: Media Resources  Media Resources group

The Media Resource Group (MRG) contains media resources, such as Conference Bridge, Transcoder, MoH server and Annunciator. It will be assigned to a Media Resource Group List (MRGL) which is used to allocate media resources to groups of devices through Device Pools, or individually by configuring a valid MRGL at the device configuration page.

Set Name*= MRG_MTP	- This is used for this example.

Set Description = MRG_MTP - This text is used to define this Media Resource Group List.

Set all Resources in the selected Media Resources Box.

## Media Resource Group List Configuration

Navigation Path: Media Resources  Media Resource Group List

Set Name = MRGL_MTP.

Set selected Media Resource Groups = MRG_MTP.

## UC Service Configuration

Navigation: User Management  User Settings  UC Service

UC Service Configuration (Contd…)

Select UC Service Type: = CTI

Set Name* = CTI_SRV. This is used in this example.

Set Description = CTI for Jabber Clients. This is used in this example.

Set Host Name/IP Address* = 10.80.14.2 (Cisco UCM Address)

UC Service Configuration (Contd…)

Select UC Service Type: = IM and Presence

Set Name* = IMP_SRV. This is used in this example.

Set Description = IM Presence. This is used in this example.

Set Host Name/IP Address* = 10.80.14.3 (Cisco UCM IM & Presence IP Address)

## Service Profile Configuration

Navigation: User Management  User Settings  Service Profile

Set Name* = Jabber_SVC_Profile. This is used in this example.

Set Description = Jabber Service Profile. This is used in this example.

Check - Make this the default service profile for the system.

Service Profile Configuration (Contd…)

## End User Configuration

Navigation: User Management  End User

Set User ID* = jabber1 – This is used in this example.

Set Password = Password for profile.

Set Directory URI = jabber1@lab.tekvizion.com.

End User Configuration (continued…)

End User Configuration (continued…)

End User Configuration(continued… )

## Cisco IP Phone 7975 SIP Configuration

Set MAC Address* = the below mac is used in this example.

Set Description = SIP-2754. This text is used to identify this Phone.

Set Device Pool*= G729 Pool. This is used in this example.

Set Phone Button Template*= Standard 7975 SIP. This is used in this example.				 Set Media Resource Group List = MRGL_MTP. This is used in this example.

Set User Hold MOH Audio Source = 1-SampleAudioSource.

Set Network Hold MOH Audio Source = 1-SampleAudioSource

Cisco IP Phone 7975 SCCP Configuration (Continued…)

Cisco IP Phone 7975 SCCP Configuration (Continued…)

Cisco IP Phone 7975 SCCP Configuration (Continued…)

Cisco IP Phone 7975 SCCP Configuration (Continued…)

Cisco IP Phone 7975 SCCP Configuration (Continued…)

Cisco IP Phone 7975 SIP Configuration (Continued…)

Set Directory Number* = 2754. This is used in this example.

Set Description = 7322162754. This is used in this example.

Set Alerting Name = 7322162754. This is used in this example.

Set ASCII Alerting Name = 7322162754. This is used in this example.

Cisco IP Phone 7975 SCCP Configuration (Continued…)

Cisco IP Phone 7975 SCCP Configuration (Continued…)

Cisco IP Phone 7975 SCCP Configuration (Continued…)

Set Display (caller ID) = 7322162753. This is used in this example.

Set ASCII Display (caller ID) = Cisco7975-Phone 1. This is used in this example.

Set Line Text Label = Cisco7975-Phone 1. This is used in this example.

Set External Phone Number Mask = 7323204350. This is used in this example.

## Cisco IP Phone 9971 SIP Configuration

Set MAC Address* = the below mac is used in this example.

Set Description = SIP-2753. This text is used to identify this Phone.

Set Device Pool*= G729. This is used in this example.

Set Phone Button Template*= Standard 9971 SIP. This is used in this example.				 Set Media Resource Group List = MRGL_MTP. This is used in this example.

Set User Hold MOH Audio Source = 1-SampleAudioSource.

Set Network Hold MOH Audio Source = 1-SampleAudioSource

Cisco IP Phone 9971 SIP Configuration(Continued…)

Cisco IP Phone 9971 SIP Configuration(Continued…)

Cisco IP Phone 9971 SIP Configuration(Continued…)

Cisco IP Phone 9971 SIP Configuration(Continued…)

Cisco IP Phone 9971 SIP Configuration(Continued…)

Cisco IP Phone 9971 SIP Configuration (Continued…)

Cisco IP Phone 9971 SIP Configuration (Continued…)

Cisco IP Phone 9971 SIP Configuration (Continued…)

Set Directory Number* = 2753. This is used in this example.

Set Description = 7322162753. This is used in this example.

Set Alerting Name = 7322162753. This is used in this example.

Set ASCII Alerting Name = 7322162753. This is used in this example.

Cisco IP Phone 9971 SIP Configuration (Continued…)

Cisco IP Phone 9971 SIP Configuration (Continued…)

Cisco IP Phone 9971 SIP Configuration (Continued…)

Set Display (caller ID) = 7322162753. This is used in this example.

Set ASCII Display (caller ID) = 7322162753. This is used in this example.

Set Line Text Label = 7322162753. This is used in this example.

Set External Phone Number Mask = 7322162753. This is used in this example.

## SIP Trunk Security Profile Configuration used by SIP trunk to Cisco UBE

Navigation: System  Security  SIP Trunk Security Profile

Set Name* = ATT Non Secure SIP Trunk Profile. This is used in this example.

Set Description = Non Secure SIP Trunk Profile authenticated by null String. This is used in this example.

Set Device Security Mode = Non Secure.

Set Incoming Transport Type* = TCP+UDP.

Set Outgoing Transport Type = UDP.

## SIP Profile Configuration used by SIP trunk to Cisco UBE

Navigation: Device  Device Settings  SIP Profile

Set SIP profile Name * = Standard SIP Profile w/Early Media Disabled. This is used for this example

Check Disable Early Media on 180

Set SIP Rel1xx Options* = Send PRACK if 1xx contains SDP

Note*= Some PSTN network call prompters utilize early-media cut-through to offer menu options to the caller (DTMF select menu) before the call is connected. In order for Cisco UCM/Cisco UBE solution to achieve successful early-media cut-through, the Cisco UCM to Cisco UBE call leg must be enabled with SIP PRACK. To enable SIP PRACK on the Cisco UCM, the SIP Profile “SIP Rel1XX Options” setting must be set to “Send PRACK”.

SIP Profile Configuration used by SIP trunk to Cisco UBE (Continued…)

SIP Profile Configuration used by SIP trunk to Cisco UBE (Continued…)

SIP Profile Configuration used by SIP trunk to Cisco UBE (Continued…)

## SIP Trunk to Cisco UBE Configuration

Navigation: Device  Trunk

Set Device Name* = ATT_SIP_TRUNK. This is used for this example

Set Description = ATT SIP Trunk to PSTN. This is used for this example

Set Device Pool* = G729_pool. This is used for this example

Set Media Resource Group List = MRGL_MTP.

SIP Trunk to Cisco UBE Configuration (Continued…)

Set Significant Digits* = 4. This is used in this example.

SIP Trunk to Cisco UBE Configuration (Continued…)

SIP Trunk to Cisco UBE Configuration (Continued…)

Set Destination Address = Set IP address of ISR-Cisco UBE.

Set SIP Trunk Security Profile* = ATT_Non Secure Sip Trunk Profile.

Set SIP Profile* = Standard SIP Profile w/Early Media Disabled. This is used in this example.

SIP Trunk to Fax Gateway Configuration.

Navigation: Device  Trunk

Set Device Name* = Trunk_SIP_FAX_Gateway. This is used for this example

Set Description = Trunk_SIP_FAX_Gateway. This is used for this example

Set Device Pool* = G729 pool. This is used for this example

Set Media Resource Group List = MRGL_MTP.

SIP Trunk to Fax Gateway Configuration (Continued…)

SIP Trunk to Fax Gateway Configuration (Continued…)

SIP Trunk to Fax Gateway Configuration (Continued…)

## Route Pattern Configuration

Navigation: Call Routing  Route/Hunt  Route Pattern

Set Route Pattern* = 9. @   This is used to route to AT&T via ISR Cisco UBE.

Set Description = To PSTN via ATT SIP Trunk. This text is used to identify this Route Pattern.

Set Gateway/Route List* = ATT_SIP_TRUNK. This is used for this example.

All other values are default

Route Pattern Configuration (Continued…)

Route Pattern Configuration (Continued…)

Route Pattern Configuration (Continued…)

Set Route Pattern* = *X!   This is used to route to AT&T via ISR Cisco UBE.

Set Description = Network-Based Call Forwarding. This text is used to identify this Route Pattern.

Set Gateway/Route List* = ATT_SIP_TRUNK. This is used for this example.

All other values are default

Note: This Route pattern is used to Activate/De-activate Network Based Call Forwarding Features.

Route Pattern Configuration (Continued…)

Route Pattern Configuration (Continued…)

Set Route Pattern* = 2753 this is used to route to Fax Client via Fax Gateway.

Set Description = To Fax Gateway. This text is used to identify this Route Pattern.

Set Gateway/Route List* = To_FaxGateway. This is used for this example.

All other values are default

Route Pattern Configuration (Continued…)

## Jabber Client Configuration

Navigation: Device  Phone

Select Phone Type* = Cisco Unified Client services framework

Set Device Name* = CSFUser1. This is used in this example.

Set Description = CSFUser1. This is used in this example.

Select Device Pool = G729. This is used in this example.

Select Phone Button Template* = Standard Client Services Framework.

Jabber Client Configuration (Contd…)

Media Resource Group List = MRGL_MTP

Set Owner check box

Set Owner user ID* = jabber1. This is used for this example

Jabber Client Configuration (Contd…)

Jabber Client Configuration (Contd…)

Jabber Client Configuration (Contd…)

Jabber Client Configuration (Contd…)

## Voicemail Port Configuration

Navigation: Advanced Feature Voice Mail  Cisco Voice Mail Port

Voicemail Port Configuration (Continued…)

Set Port Name = CiscoUM1-VI1. This is used for this example.

Set Description = VM Port. This is used for this example.

Set Device Pool = G729

Set Directory Number* = 2501. This is used in this example.

## Voicemail Pilot Configuration

Navigation: Advanced Features  Voice Mail  Voice Mail Pilot

Set Voice mail Pilot Number = 2303. This is used for this example

Set Description = VoiceMail Pilot number with SIP

# FAX Gateway Configuration

cme.in.tekvizion.com#sh version

Cisco IOS Software, 2800 Software (C2800NM-IPVOICEK9-M), Version 15.1(4)M5, RELEASE SOFTWARE (fc1)

Technical Support: http://www.cisco.com/techsupport

Copyright (c) 1986-2012 by Cisco Systems, Inc.

Compiled Tue 04-Sep-12 15:56 by prod_rel_team

ROM: System Bootstrap, Version 12.4(13r)T, RELEASE SOFTWARE (fc1)

cme.in.tekvizion.com uptime is 1 day, 5 hours, 59 minutes

System returned to ROM by reload at 14:27:25 IST Sun Jan 10 2016

System image file is "flash:c2800nm-ipvoicek9-mz.151-4.M5.bin"

Last reload type: Normal Reload

This product contains cryptographic features and is subject to United

States and local country laws governing import, export, transfer and

use. Delivery of Cisco cryptographic products does not imply

third-party authority to import, export, distribute or use encryption.

Importers, exporters, distributors and users are responsible for

compliance with U.S. and local country laws. By using this product you

agree to comply with applicable laws and regulations. If you are unable

to comply with U.S. and local laws, return this product immediately.

A summary of U.S. laws governing Cisco cryptographic products may be found at:

http://www.cisco.com/wwl/export/crypto/tool/stqrg.html

If you require further assistance please contact us by sending email to

export@cisco.com.

Cisco 2851 (revision 1.0) with 249856K/12288K bytes of memory.

Processor board ID FHK1137F4LY

2 Gigabit Ethernet interfaces

62 Serial interfaces

2 terminal lines

2 Channelized E1/PRI ports

4 Voice FXS interfaces

2 cisco service engine(s)

DRAM configuration is 64 bits wide with parity enabled.

239K bytes of non-volatile configuration memory.

62720K bytes of ATA CompactFlash (Read/Write)

License Info:

License UDI:

-------------------------------------------------

Device#   PID                   SN

-------------------------------------------------

*0        CISCO2851             FHK1137F4LY

Configuration register is 0x2102

cme.in.tekvizion.com#sh running-config

Building configuration...

Current configuration : 7689 bytes

!

! Last configuration change at 14:02:24 IST Mon Mar 28 2016 by cisco

version 15.1

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname cme.in.tekvizion.com

!

boot-start-marker

boot-end-marker

!

!

enable password tekV1z10n

!

aaa new-model

!

!

aaa authentication login local_auth local

!

!

!

!

!

aaa session-id common

clock timezone IST 5 30

network-clock-participate wic 2

network-clock-participate wic 3

!

dot11 syslog

ip source-route

!

!

ip cef

!

!

!

ip host Clus1-862-Pub 172.16.26.2

no ipv6 cef

multilink bundle-name authenticated

!

!

!

!

isdn switch-type primary-qsig

!

!

voice rtp send-recv

!

voice service pots

!

voice service voip

no ip address trusted authenticate

allow-connections sip to sip

redirect ip2ip

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

sip

midcall-signaling passthru

g729 annexb-all

!

voice class codec 2

codec preference 1 g711alaw

codec preference 2 g729r8

!

voice class codec 1

codec preference 1 g729r8

codec preference 2 g711alaw

!

!

voice class sip-profiles 1

response ANY sip-header Allow-Header modify "UPDATE," ""

request ANY sip-header Allow-Header modify "UPDATE," ""

response ANY sip-header Allow-Header modify "UPDATE," ""

response ANY sip-header Allow-Header modify "UPDATE," ""

!

!

!

!

!

voice-card 0

!

crypto pki token default removal timeout 0

!

!

!

!

license udi pid CISCO2851 sn FHK1137F4LY

username cisco password 0 tekV1z10n

!

!

controller E1 0/2/0

pri-group timeslots 1-31 service mgcp

!

controller E1 0/3/0

clock source internal

pri-group timeslots 1-31

!

ip tftp source-interface GigabitEthernet0/0

!

!

!

!

!

!

!

!

!

interface GigabitEthernet0/0

ip address 172.16.31.50 255.255.255.0

duplex auto

speed auto

!

interface Service-Engine0/0

no ip address

shutdown

!

interface GigabitEthernet0/1

no ip address

ip nat outside

ip virtual-reassembly in

shutdown

duplex auto

speed auto

!

interface Serial0/2/0:15

no ip address

encapsulation hdlc

isdn switch-type primary-qsig

isdn timer T310 120000

isdn protocol-emulate network

isdn incoming-voice voice

isdn map address .* plan isdn type national

isdn bind-l3 ccm-manager

isdn send-alerting

isdn sending-complete

no cdp enable

!

interface Serial0/3/0:15

no ip address

encapsulation hdlc

isdn switch-type primary-qsig

isdn timer T310 120000

isdn protocol-emulate network

isdn incoming-voice voice

no cdp enable

!

interface Service-Engine1/0

no ip address

shutdown

!

ip forward-protocol nd

!

ip http server

no ip http secure-server

!

ip route 0.0.0.0 0.0.0.0 172.16.31.1

ip route 0.0.0.0 0.0.0.0 172.16.29.1

!

!

snmp-server community public RO

snmp-server location Chennai

!

!

!

!

!

control-plane

!

!

voice-port 0/0/0

ring frequency 50

no echo-cancel enable

no vad

cptone IN

station-id name fax test

station-id number 7322162753

caller-id enable

!

voice-port 0/0/1

no vad

shutdown

!

voice-port 0/3/0:15

!

voice-port 0/2/0:15

!

voice-port 0/1/0

no vad

shutdown

!

voice-port 0/1/1

no vad

shutdown

!

no mgcp timer receive-rtcp

mgcp behavior rsip-range tgcp-only

mgcp behavior comedia-role none

mgcp behavior comedia-check-media-src disable

mgcp behavior comedia-sdp-force disable

!

mgcp profile default

!

!

!

!

!

!

dial-peer voice 110 pots

service session

destination-pattern 2753

no digit-strip

port 0/0/0

forward-digits all

!

dial-peer voice 200 voip

description CUCM to Gateway

service session

session protocol sipv2

session transport udp

incoming called-number 2753

voice-class codec 1

voice-class sip profiles 1

dtmf-relay rtp-nte

fax rate 14400

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

no vad

!

dial-peer voice 201 voip

description Gateway to CUCM

service session

destination-pattern 9T

session protocol sipv2

session target ipv4:10.80.14.2

session transport udp

voice-class codec 1

voice-class sip profiles 1

dtmf-relay rtp-nte

fax rate 14400

fax protocol t38 version 0 ls-redundancy 0 hs-redundancy 0 fallback none

no vad

!

!

gateway

timer receive-rtp 1200

!

sip-ua

credentials username +19728522671 password 7 11584B5643475D realm bsrhelas.lab.tekvizion.com

no remote-party-id

retry register 5

timers connection aging 30

timers update 1000

no timers hold

timers register 1000

protocol mode dual-stack

!

!

telephony-service

max-ephones 50

max-dn 60

ip source-address 172.16.31.50 port 2000

service phone sshAccess 0

cnf-file perphone

max-conferences 8 gain -6

web admin system name Administrator password tekV1z10n

transfer-system full-consult

create cnf-files version-stamp 7960 Nov 22 2013 19:05:58

!

!

ephone-dn  1

!

!

ephone-dn  2

!

!

ephone-dn  5

!

!

ephone-dn  6  dual-line

!

!

ephone-dn  10

!

!

ephone  1

mac-address 001D.A21A.3577

type 7961

button  1:1

!

!

!

ephone  2

mac-address 001D.A21A.291D

type 7961

button  1:2

!

!

!

ephone  5

mac-address 0008.21DF.CEC5

type 7940

ssh userid cisco password cisco

button  1:5

!

!

!

ephone  6

!

!

!

ephone  10

mac-address 0C27.2431.5FB9

button  1:10

!

!

!

banner login ^CC

=================================================

WELCOME to CISCO CME 8.6

=================================================

Cisco IOS Software (C2800NM-IPVOICEK9-M)

Version 15.1(4)M5, RELEASE SOFTWARE (fc1)

Cisco 2851 with 249856K/12288K bytes of memory

Processor board ID FHK1137F4LY

2 Gigabit Ethernet interfaces

2 terminal lines

2 Voice FXS interfaces

2 cisco service engine(s)

239K bytes of non-volatile configuration memory

62720K bytes of ATA CompactFlash (Read/Write)

Warning: Access is restricted.

All user activity is logged!

For support, contact 'kkumaraguru@tekvizion.com'

=================================================

^C

!

line con 0

line aux 0

line 66

no activation-character

no exec

transport preferred none

transport input all

transport output pad telnet rlogin lapb-ta mop udptn v120 ssh

line 194

no activation-character

no exec

transport preferred none

transport input all

transport output all

line vty 0 4

session-timeout 180

exec-timeout 0 0

password tekV1z10n

login authentication local_auth

transport input all

!

scheduler allocate 20000 1000

ntp server 103.6.16.254

end

# Cisco UCM SCCP Integration with Cisco Unity Connection (CUC)

## CUC Version

## CUC Telephony Integration with Cisco UCM

Navigation: Telephony Integrations  Phone system

Set Phone System Name* = SIP. This is used for this example

## CUC Port Group

Navigation: Telephony Integration  Port Group

CUC Port Group(continued…)

Set Display Name* = SIP-1. This is used in this example.

Check Register with SIP Server.

## CUC Port Settings

## CUC Sample User Basic Settings

Navigation: Cisco Unity connection  Users  Users

Set Alias = 4051.This is one of the extension used for this testing.

Set Extension = 4051. This is used for this example.

CUC Sample User Basic Settings (Continued…)

Set Partition = clus24-unity partition. This is used for this example.

Select Search Scope = clus24-unity Search Scope.

Select Phone System = SIP.

## Auto Attendant

Navigation:  Call Management  System Call Handlers

Set Display Name = Demo auto attend. This is used for this example.

Set Phone System = SIP

Set Extension=2999. This number is used as Auto attendant on this set up.

Set Partition = Clus24-unity Partition. This is used for this example.

Auto Attendant (Continued…)

# Cisco UCM Integration with Cisco Unified CM IM and Presence (CUP/IMP)

## CUP/IMP Version

## Presence Topology

Navigation: System  Presence Topology

## Node Configuration

Navigation: System  Presence Topology  Fully Qualified Domain Name

## Users

Navigation: System  Cluster Topology  clus24imp.lab.tekvizion.com  Users

## Presence gateway configuration

Navigation: Presence  Gateways

Set Presence Gateway Type *= CUCM

Set Description *= Cluster 24. This is used for this example.

Presence Gateway * =clus24pubsub.lab.tekvizion.com

# Acronyms

| AVPN; AT&T Virtual Private Network. CODEC; Coder-Decoder (in this document a device used to digitize and undigitize voice signals). Cisco UBE; Cisco Unified Border Element. Cisco UCM; Cisco Unified Communications Manager. IP; Internet Protocol. ISR; Integrated Services Router. MGCP; Media Gateway Control Protocol. MIS; Managed Internet Services. PNT; Private Network Transport. PSTN; Public switched telephone network. SCCP; Skinny Client Control Protocol. SIP; Session Initiation Protocol. SP; Service Provider. TDM; Time-division multiplexing. |  |
| --- | --- |

# Important Information

THE SPECIFICATIONS AND INFORMATION REGARDING THE PRODUCTS IN THIS MANUAL ARE SUBJECT TO CHANGE WITHOUT NOTICE. ALL STATEMENTS, INFORMATION, AND RECOMMENDATIONS IN THIS MANUAL ARE BELIEVED TO BE ACCURATE BUT ARE PRESENTED WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED. USERS MUST TAKE FULL RESPONSIBILITY FOR THEIR APPLICATION OF ANY PRODUCTS. IN NO EVENT SHALL CISCO OR ITS SUPPLIERS BE LIABLE FOR ANY INDIRECT, SPECIAL, CONSEQUENTIAL, OR INCIDENTAL DAMAGES, INCLUDING, WITHOUT LIMITATION, LOST PROFITS OR LOSS OR DAMAGE TO DATA ARISING OUT OF THE USE OR INABILITY TO USE THIS MANUAL, EVEN IF CISCO OR ITS SUPPLIERS HAVE BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

| Corporate Headquarters Cisco Systems, Inc. 170 West Tasman Drive San Jose, CA 95134-1706 USA www.cisco.com Tel: 408 526-4000 800 553-NETS (6387) Fax: 408 526-4100 | European Headquarters CiscoSystems International BV Haarlerbergpark Haarlerbergweg 13-19 1101 CH Amsterdam The Netherlands www-europe.cisco.com Tel: 31 0 20 357 1000 Fax: 31 0 20 357 1100 | Americas Headquarters Cisco Systems, Inc. 170 West Tasman Drive San Jose, CA 95134-1706 USA www.cisco.com Tel: 408 526-7660 Fax: 408 527-0883 | AsiaPacific Headquarters Cisco Systems, Inc. Capital Tower 168 Robinson Road #22-01 to #29-01 Singapore 068912 www.cisco.com Tel: +65 317 7777 Fax: +65 317 7799 |
| --- | --- | --- | --- |
| Cisco Systems has more than 200 offices in the following countries and regions. Addresses, phone numbers, and fax numbers are listed on the Cisco Web site at http://www.cisco.com/go/offices. Argentina • Australia • Austria • Belgium • Brazil • Bulgaria • Canada • Chile • China PRC • Colombia • Costa Rica • Croatia • Czech Republic • Denmark • Dubai, UAE • Finland • France • Germany • Greece • Hong Kong SAR • Hungary • India • Indonesia • Ireland • Israel • Italy • Japan • Korea • Luxembourg • Malaysia • Mexico • The Netherlands • New Zealand • Norway • Peru • Philippines • Poland • Portugal • Puerto Rico • Romania • Russia • Saudi Arabia • Scotland • Singapore • Slovakia • Slovenia • South Africa • Spain • Sweden • Switzerland • Taiwan • Thailand • Turkey Ukraine • United Kingdom • United States • Venezuela • Vietnam • Zimbabwe © 2016 Cisco Systems, Inc. All rights reserved. CCENT, Cisco Lumin, Cisco Nexus, the Cisco logo and the Cisco Square Bridge logo are trademarks of Cisco Systems, Inc.; Changing the Way We Work, Live, Play, and Learn is a service mark of Cisco Systems, Inc.; and Access Registrar, Aironet, BPX, Catalyst, CCDA, CCDP, CCVP, CCIE, CCIP, CCNA, CCNP, CCSP, Cisco, the Cisco Certified Internetwork Expert logo, Cisco IOS, Cisco Press, Cisco Systems, Cisco Systems Capital, the Cisco Systems logo, Cisco Unity, EtherFast, EtherSwitch, Fast Step, Follow Me Browsing, FormShare, GigaDrive, HomeLink, Internet Quotient, IOS, iPhone, iQ Expertise, the iQ logo, iQ Net Readiness Scorecard, iQuick Study, LightStream, Linksys, Meeting Place, MGX, Networking Academy, Network Registrar, Packet, PIX, ProConnect, ScriptShare, SMARTnet, StackWise, The Fastest Way to Increase Your Internet Quotient, and TransPath are registered trademarks of Cisco Systems, Inc. and/or its affiliates in the United States and certain other countries. All other trademarks mentioned in this document or Website are the property of their respective owners. The use of the word partner does not imply a partnership relationship between Cisco and any other company. (0705R) Printed in the USA |  |  |  |

Hide signaling and media peer addresses from endpoints other than gateway.

If the mode border-element command is not entered, border-element-related commands are not available for Cisco Unified Border Element voice connections on the Cisco 2900 and Cisco 3900 series platforms with a universal feature set. The mode border-element command is not available on any other platforms.

This command enables Cisco UBE basic IP-to-IP voice communication feature.

This command allows SIP error messages to pass-through end-to-end without modification through       Cisco UBE.

This command enables delay offer-to-early offer conversion of initial SIP INVITE message to calls matched to this dial-peer level.

This command must be enabled at a global level to maintain integrity of SIP signaling between AT&T network and Cisco Unified Communications Manager (Cisco UCM) across Cisco UBE.

This command allows for privacy settings to be transparently passed between AT&T network and Cisco UCM. This command can either be set at a global level, such as in this example, or it can be set at the dial-peer level.

This command configures the codec preference to be assigned to dial-peers. Alternatively, single code can be configured into individual dial-peers.

This SIP profile expands the Diversion header number from a 4-digit extension to a full 10-digit DID number in order to obtain interoperability with AT&T’s served users during call-forward scenarios. The six digits in "sip: 732216" are variable and must be replaced with the first 6 digits of the DID's provisioned for the customer site.

Cisco 6900-series IP phones use ptime value of 20 ms. AT&T networks prefer ptime value of 30 ms. This SIP profile modifies SDP ptime value from 20 to 30 ms and it should be applied to dial-peers where G729 is the preferred codec.  If the customer creates a dial-peer specifically for G711, a sip-profile without modifying the ptime value should be applied. This is because G711 RTP was not defaulting to 20ms.

This SIP profile is required in order to advertise the ptime=30 attribute in the outgoing SIP INVITE from Cisco UBE to AT&T. Currently RFC’s do not have a standard method to advertise ptime values for each offered codec within a SDP offering with multiple codecs. This SIP profile allows for Cisco UBE to include the ptime attribute with a value of 30ms.

This command used to convert 4 digit to 10 digit in contact header otherwise ATT will send 6xx error response while executing network related feature.

WAN interface to AT&T

LAN interface to Cisco UCM

Cisco UBE LAN interface IPv4 Address

Dial peer for AT&T facing network

Session protocol SIPv2 is used for this testing

Assigns voice class codec 1 settings to dial-peer (codec support and filtering).

Configures the dynamic SIP asymmetric payload support.

This command allows for privacy settings to be transparently passed between AT&T network and Cisco UCM. In this example, the command is set at the dial-peer level, you can also set the command at a global level to affect all dial-peers without necessarily setting the command on each dial-peer.

This command enables the dial peer to use SIP profile 1

Configure the Cisco UBE SIP messaging to use the HSRP virtual address in SIP messaging. Once HSRP is configured under the physical interface and the bind command is issued, calls to the physical IP address will fail. This is because the SIP listening socket is now bound to the virtual IP address but the signaling packets use the physical IP address, and therefore cannot be handled.

This command used to pass RTP NTE (RFC2833) DTMF with respect to the dial peers used for the call.

This command enables T38 fax protocol for calls terminating on this dial-peer